https://www.ti.com/lit/an/slva504a/slva504a.pdf?ts=1771859867799#:~:text=PRON%20%5BW%5D%20=%20RON%20%C3%97,diode%20forward%20bias%20voltage%20%5BV%5D

To do

- Learn about motor MOSFETS based PWM generation techniques

- Control PWM signal for full bi-directional speed control of motor

- Current limiters + voltage conditioning => so PMW controlling doesn't damage motor or microcontroller

- Design brake by reducing input to the drive PMW

- Decoupling capacitors on power supply rail to reduce inteference with other subsystems (motor and driver circuits loads of EM noise and power supply ripple)

- Baseline motor subsystem – no decoupling

- Study components of circuits ( which contribute to what behaviour)

  - H-bridge – for complementary switching and back EMF protection

  - Motor

  - Other things lol

- Model motor as R-L (resistor inductor) series circuit

  - low R_motor (5 to 30 Ohms max.)

  - L_motor as an inductance (sweep from 50uH to 50 mH)

  - back EMF as a current-dependent DC source (i.e. current proportional to speed of the motor).

- Build circuit ( LTSpice -> project board)

- Add power decoupling of the (PIC) rectangular pulses

  - Gate drive circuit between h bridge + Pic output

  - Decoupling of motor noise into DC supply

- Study :

  - transient behaviour to reason for component functionality and back EMF behaviour with different RL characteristics of the DC motor.

  - Steady-state behaviour for investigating the overall circuit behaviour.

  - AC sweep of the input source to reason for performance of your

- switching signals (coming from the controller, e.g., rectangular pulses).

  - Also create a model for relating your DC motor outputs based on the rectangular PWM ratios. This model will be crucial for your integration plans.

  - DC sweep to investigate the minimum DC operating. point (especially the battery is low).

  - Fault diagnostics capability with simulated open and short circuit faults at different points in the circuit.

  - What would the circuit behave without back EMF protection?

  - What is the impact of motor noise decoupling?

  - Can you measure the power consumption (V x I) for different PWM duty cycles and report back to power subsystems for load modelling?

H- bridge - https://www.modularcircuits.com/blog/articles/h-bridge-secrets/h-bridges-the-basics/

Brushed motors generally operate at low speed and can be driven by a simple pulse width modulation (PWM) controller to vary the voltage supplied to the motor to control the speed in one direction and provide the torque for the motor drive.

Q1,q3 – p mosfet

Q2,q4 – n mosfet

q1+q4= power motor foward, q2,a3 revers4 ucrrent flow, driving it backwards Closing bot high side(q1,q2) or lowside(3,4)= fast stop, but opeing all allows to coast)

NEVER CLOSE Q1 AND Q2 OR Q3 AND Q4

MOTOR CAN BE SHOWN AS RESITOR INDUCTOR AND Vg(movement of inducotr in field creats a volatge

The generator voltage (Vg) depends only on the speed by which the inductors move in the field, in other words on the rotational speed of the motor.

Pwm signal  - on and off time – constant frequency. Info content = on:off ratio

Cant change the current thrugh an inductor instantaneously, so if bridg changes state – new state must make sure current can ekep flowing, the state after change in pwm musthae a path  for curren tto flow out

Catch diodes to provide path for current in the short switching time

Use

Switch over diode while current is lower tgab vf(forward volatge/i)

Birectional dc motor drive:  Take pwm from pic , drive motor foward backwards

Input = pwm from pic (0-5v), direction control signal, battery

Output = motor volatge/current , power consumption stuff

Constraints – limit motor current, no back emf damanfe, Handle excessive noise

Pmw from pic-> gate driver stage->hbridge->motor model (R-L +back emf)-> current lmiiting-> decoupling and portection

1)Build h bridge on lt spice (simulate how motor would move , forward,back,brake and stop) + smiluate unforseen circmustances

2) pwm + gate drive cirucitry to turn mosfet on and off ( take pic pwm , supply higher current to mosfet gate + switches cleanly and quickly, e.g transistor buffer , IC, pushpullstage?)

3) current limiting stops motor from draing too muc current at startp + frying lol (fix withmaybe  a small reistor in seires withmotor / resarch hardare current cutoff (if v> cutoff) - simuate in lt spice with normal op, increase load at startup, short circuit – if no current limting mosfet linear, p increases very fast (thermal runaway)

4) power decoupling and noise  ( capacitor near Ic power pin + near motor supplyto stabilise volatge and minimise noise

5) ensure all simulations:sudden changes ( startup, direction change, braking), steady state ( constant pwm duty cycle, and current ripple), Ac sweep ( sweep of pwmfrquencies and how it affects current ripple (find out more about that lol, find suitabl frequncy) dc sweep (cary supply volatge , battery volatge rop,lowest op for operation=), fault when no

Lt spice

When you attach a cursor:

- X-value = time

- Y-value = voltage

https://www.mathworks.com/help/sps/ug/pwm-controlled-dc-motor.html

ontrol DC Motor with PWM Voltage Source and H-Bridge Driver component

By varying the width of these digital pulses, the ratio of the “ON” time (pulse width) to the total period T (On + OFF time), determines the percentage duty cycle of the pulses within a fixed output frequency. That is: Duty Cycle (%) = (Pulse Width / T)*100.

Duty cycle is commonly expressed as a percentage. So a 10% duty cycle means the signal is ON 10% of the time, while a 90% duty cycle means it’s ON for 90% of the time and OFF for 10%. Clearly, a 50% duty cycle means the signal is ON for half the time and OFF for the other half producing a square wave signal.

The average voltage and therefore power delivered to the motor load is calculated as: Vaverage = duty cycle times VMAX.

Then, Pulse width modulation is a great method of controlling the amount of power delivered to a load without dissipating any wasted power. The above circuit can also be used to control the speed of a fan or to dim the brightness of DC lamps or LED’s. If you need to control it, then use Pulse Width Modulation to do it.

https://www.youtube.com/watch?v=Wc4XzTrWSpo

- MODEL BRIUSHED MOTOR

https://www.youtube.com/watch?v=yO9xIVv8ryc

WHAT IS A BRUSHED DC MOTORR

https://www.youtube.com/watch?v=htdEn1Eyvcw

# Simulation of RL (resistor-inductor) circuits in LT spice

https://www.youtube.com/watch?v=S88fnMSA-aM

https://www.youtube.com/watch?v=DXyTHhUjxjk

driver gate

Slow stiching time -> swithcing losses

Gat ediver enables low volatrge (when pwm is supe rlow

Transient protection when switching

Fast swithving

- High-Side Switches: MOSFETs or BJTs connected to the positive supply (v motor

- Gate Driving: Because the source voltage of a high-side N-channel MOSFET rises to v m motor

, the gate voltage must be higher than this to turn on, necessitating a dedicated high-side driver IC (e.g., IR2110).

- Bootstrap Circuits: A capacitor (c boot) and diode are used to generate the necessary high-side gate voltage, which requires a minimum PWM duty cycle to function properly (cannot run at 100% duty cycle).

First attempt at hbridge and motor – the vdd biases so i removed inverter

https://bitdrivencircuits.com/hBridge/hbridge_assembly.html

WHILE TESTING THE ltspice, i used nmmos and cmos with simialr values ot the datasheetthius should be 0 v – find otu why its not 0

This should be the same as vdd – is work sas its 7.2 v